A COMPLETE ENGINE REPAIR

JOURNAL GRINDING, ANNEALING, LASER

ALIGNMENT, LINE BORING AND ENGINE

REBUILD

Goltens was presented with a challenging opportunity . The customer’s engine had incurred significant

damage to the crankshaft and block. Goltens’ technicians

performed an evaluation of the shaft and engine and found

that the crankshaft was damaged on one Main journal and

one Crankpin journal and hardness tests showed areas in

both journals around 600 Brinell. In order to save the shaft, it

would need to be annealed. Additionally, the main journal

pocket was damaged by the main journal bearing failure and

had to be line bored and so the crankshaft was removed

from the engine.

The customer did not

want to incur the costs of shipping the shaft overseas to be

reconditioned in the workshop and requested that Goltens

perform all repairs on site.

Goltens mobilized Diesel technicians and tools to

disassemble the engine and remove the crankshaft.

Simultaneously Goltens mobilized in-situ line boring,

crankshaft grinding, annealing equipment and technicians to

site

REPAIRS CONSISTED OF:

Diameter measurement, Magnaflux and Hardness testing

of crankshaft

Grinding of Main and Crankpin journal until free of cracks

Disassembly of engine and removal of crankshaft

Annealing of both journals to remove excessive hardness

Laser bore alignment check of all main journal bores

Machining all lower bearing caps

Line Boring of all main bearing pockets to standard size

Reinstallation of crankshaft and reassembly of the

engine block

Finish grinding of the crankshaft to 2.00mm and 3.00mm

undersize for main and crankpin journals respectively

Reassembly of all engine components, completion of

pre-start checks and operational run in of the engine

RESULTS:

Goltens technicians completed the entire engine repair onsite

using its full capabilities as specialist engine repair

engineers and In-Situ machining specialists. The engine was

restored to full operation and the customer avoided the cost

of transporting the damaged components to a remote location.